Technical Datasheet

Date： 2022.1.13 Version: 1.0 Product： 2-Methylcyclohexylamine CAS-No.: 7003-32-9

Descriptions：

Colorless to light yellow transparent liquid at room temperature, soluble in alcohol and ether solvents, insoluble in water.It's a strong basic tertiary amine compound. Applications：. It is mainly used as a catalyst for rigid polyurethane foam. It is a medium active amine catalyst with low viscosity, used for refrigerator, plate, spraying and spot perfusion of rigid polyurethane foam..

| Items can be detected |  |
| --- | --- |
| Test Item | Specification |
| Purity，%min | 99.0 |
| Water content, %max | 0.5 |

| Typical Properties |  |
| --- | --- |
| Appearance | Colorless to light yellow liquid |
| Flash point | 40-41℃ |
| Boiling point | 149-1509ºC |
| Freezing point | -60ºC |
| Density | 0.85-0.87g/cm3 |
| Water solubility | slightly soluble |

Storage Information:

Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage. Store in cool place. Avoid contact with skin and eyes. Avoid inhalation of vapor or mist.

Shelf Life: 12 months